Applicant: _________________________  Road Name: _____________________________ Date: _____________

Applicant Reps: _______________________________________________________________________________

CD Reps: _____________________________________________________________________________________

Contractors Present: ___________________________________________________________________________

Others Present: _______________________________________________________________________________

Pre-Construction Site Visit Checklist							 D&GR        LVR

Logistical Discussion Points

- Proposed start date: _______________		Proposed completion date:______________

- CD Notification:  _________days before project work begins by applicant

- Timelines (mobilization, demobilization, project phases)

- Project Overview / Changes: (overview workplan, procedures for changes in project scope, timeline, cost)

- Notifications (PA One-Call, 911 services, notify impacted landowners, road closures, signage)

- Site Marking (Considerer painting or flagging features on the road before or during the meeting)

- Level of on-site CD involvement/oversite (at a minimum, the District should be on site when the project starts, and whenever a new practice or phase of the project begins)

- Receipts (must be kept for reimbursement, including time and equipment usage)

- Final inspection (plans for project closeout upon completion of work)

Project-Specific Discussion Points

- Erosion and sediment controls (what is needed and who is responsible)

- Landowner agreements (signed for work outside ROW)

- Staging or storage areas (for equipment and materials storage if necessary)

- Crosspipes (review location, angle, depth, fill for cover, transitions, and headwalls/endwalls/aprons)

- ESMP locations (review location and expectations for other ESM Practices such as mattresses, underdrain, etc.)

Fill Projects

- Place and compact in lifts (fill should compacted in layers no thicker than 12 inches)

- Access road connectivity (transitions to intersections, lanes, and driveways)

DSA Projects

- Testing (has DSA been tested and met SCC specifications)

- Pre-placement preparations for DSA

- Drainage/base stability (all drainage and base work is done, new fill adequately settled)

- Crowned (road base crowned or side sloped at 4-6%)

- Paving Prep (Keys required where placement starts/stops, paving notch recommended along road if possible)

- Weather (seasonal limitations, and contingencies for weather related issues)

- Delivery Logistics (# of trucks; turnaround points; spec sheet accompanies first 
load; weigh slips and receipts, optimum moisture; truck tarps; quarry contact:___________________________)

Stream Crossing Projects

- Permits (in place before work begins, meet any seasonal restrictions)

- Construction plan (timeline, traffic, pump-around or bypass for stream)

- Stream simulation (elevation, slope, grade control, and replicating stream through structure)

Notes: ___________________________________________________________________________________________ ________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________